# 🧩 Client Case — Beijing LN New Energy Co., Ltd.

YUEDU SL800 Carbon Fiber Explosion-Proof Sealed Shredding System

| Carbon Fiber Paper（碳纤维纸） | Carbon Fiber Felt（碳纤维毡） |
| --- | --- |

🧭 Explosion-Proof Sealed Carbon Fiber Shredding System — Confidential Schematic

## 1. Company Background

LN New Energy focuses on R&D and manufacturing of new-energy application materials, tackling bottleneck core materials in the hydrogen energy industry. Its carbon fiber paper and felt are used in hydrogen energy, energy storage, and high-performance composites. The team has achieved end-to-end breakthroughs in carbon-fiber application materials and notable results in GDL manufacturing and fuel-cell components.

## 2. Customer Requirements (Summary)

- Solution: Carbon-fiber shredder / explosion-proof sealed shredding system

- Materials: Carbon Fiber Paper, Carbon Fiber Felt

- Feed width: 1.2 m

- Footprint: ≤ 3 m²

- Special requirements: Dust- and explosion-proof, antistatic, fully sealed; minimize airborne dust; compact/easy maintenance; safe feeding and bagging; IP58 protection level.

## 3. Customer Pain Points & Technical Challenges

### Summary (one-glance view)

| Pain Point | YUEDU Solution |
| --- | --- |
| Strict particle control; avoid powdering | Innovative shredding approach; precise blade-gap control |
| Full sealing / dust & explosion-proof | Negative-pressure sealing + explosion-proof motor & fan (ExdⅡBT4) |
| Dust escape at collection; unsafe for operators | Sealed collection bin + negative-pressure bagging + auto sealing |
| Conductive dust damages motors/electrics | IP58 protection system + antistatic discharge path |

### Detailed points (as in the doc)

- Tight particle size (10–20 mm flakes/fibrous), no over-pulverization.
Measures: new structure to avoid high-speed shearing; precise blade gap; anti-recut discharge channel.

- Extreme sealing to prevent dust leakage & health risks.
Measures: fully sealed chamber + dust collection; sealed connections across shredder, fan, cyclone, and filter; all electrics explosion-proof; system protection IP58.

- Unsafe collection if unsealed.
Measures: sealed collection chamber with quick bag change; negative-pressure bagging; optional auto sealing & vacuum.

- Electrical damage risk from conductive carbon fiber dust.
Measures: IP58 explosion-proof design for motor/fan/electrics; antistatic conduits and sealed interfaces; isolated electrical bay with gaskets; dedicated grounding & static discharge path.

## 4. YUEDU Shredding System Solution

- Model: Customized SL800 Explosion-Proof Sealed Carbon-Fiber Shredding System

- Process flow: Feeding → Shredding → Conveying → Separation → Filtration → Collection

Explosion-Proof Sealed Carbon Fiber Shredding System — Confidential Schematic.

## 5. System Highlights

- Full sealed structure; dust- & explosion-proof; no leakage.

- Precise particle control, preserving carbon-fiber structure.

- Antistatic discharge design to prevent short circuits.

- Sealed, safe bagging to protect operators.

- Modular for easy install/maintenance; ≤ 3 m² footprint.

## 6. Quotation & Delivery

- Project: Customized SL800 explosion-proof sealed carbon-fiber shredding system — Qty: 1 set (pricing per commercial quote).

- Lead time: 20–30 working days.

- Includes: factory testing, explosion-proof certification, operation manual.

## 7. Expected Outcomes

- 10–20 mm particle size with preserved fiber structure.

- Fully sealed & explosion-proof operation; no dust pollution.

- Stable equipment operation; motors/electrics protected from conductive dust.

- Green, low-pollution, high-safety production goals achieved.

### 🌐 Contact Us

Shanghai YUEDU Technology Co., Ltd.

📞 Contact Us to explore more customized models and tailored shredding solutions that suit your specific needs!
Let us help you find the perfect equipment for your application.

📱 Sales Supervisor: +86 159 0178 2779 (WhatsApp / WeChat / Facebook)
📱 Administrative Supervisor: +86 137 6138 5116

📧 Email: yuedushredders@gmail.com | denghao8410@gmail.com 
🌐 Website: www.yuedushredders.com

@YUEDUShredders – Empowering Global Industries with Intelligent Destruction Systems.